SPECIFICATIONS OF THE TUGS, REQUIREMENT OF 1x50 T BP:

| a) |  | Length Over All (LOA) |  |  |  | 30 to 40 Mtrs |
| --- | --- | --- | --- | --- | --- | --- |
| b) |  | Draft |  |  |  | Maximum 5.5 Mtrs |
| c) |  | Bollard Pull |  |  |  | BP certificate for minimum 50T for 50Ton tug at 100% MCR from the classification society must not be more than six months old. Older tugs need to repeat this test every 2.5 years as per the DG guidelines and/or as instructed by the port authorities. |
| d) |  | Propulsion |  |  |  | Steerable rudder propulsion or Voith Schneider system or ASD or Z Peller. |
| e) |  | Free Running Speed |  |  |  | At least 12 Knots continuous with full power and under normal weather conditions. |
| f) |  | Age of tug |  |  |  | No more than 10 years. Relaxation can be granted upon satisfactory inspection of vessel, its machinery & past records. |
| g) |  | Main Engine |  |  |  | Twin engines operating on diesel with independent remote control operation or similar flexible arrangement. |
| h) |  | External Fire Fighting System |  |  |  | For external fire-fighting either fire pump or GS pump or engine coupled fire pump with one/two foam/ water monitors on monkey island in fully operational condition. Cubic capacity for external fire-fighting to be commensurate with requirement of one/two foam/water monitors installed at monkey island. |
| i) |  | Diesel Generators |  |  |  | Minimum 2 nos. of DG sets of adequate capacity required. |
| j) |  | Auxiliaries |  |  |  | For internal & external fire-fighting, either Fire Pump or GS Pump or Engine coupled fire pump is required. Tugs to maintain adequate stock of OSD & foam compound. |
| k) |  | Towing Arrangement |  |  |  | Quick release tow hook and towing winch with adequate strength and suitably fendered so as to enable the tug to push/pull as required. |
| l) | Domestic FW & SW System |  |  |  | Pressurized Hydrophore system or equivalent. |  |
| m) | Fenders |  |  |  | Tug shall be provided with strong and adequate rubber fenders at bow, stern and all along the length. Prior inspection will be carried out before acceptance. |  |
| n) | Wheel house |  |  |  | Steel with all round visibility |  |
| o) | Classification |  |  |  | By recognized society |  |
| p) | VHF set |  |  |  | 2 Nos. (Should have all Marine Channels) |  |
| q) | Bridge |  |  |  | All major machines should have Emergency/Supplementary control from the bridge. |  |
| r) | Navigational Equipment |  |  |  | Marine Radar, AIS, Echo sounder, Search Light, GPS, Navtex etc. |  |

| s) | Manning |  | As per the statuary requirement. (i) Working hours for the tug is round the clock on all days. (ii) The operator shall however arrange to keep standby staff to meet contingencies such as sudden sickness, absenteeism, leave reserve etc., for which no extra payment will be made. |
| --- | --- | --- | --- |
| u) | Safety Devices |  | Should have appropriate safety devices to work in the close vicinity of vessels carrying inflammable/hazardous cargo. |
| v) | Registration |  | Under the Merchant Shipping Act, 1958 or equivalent as a sea going tug. |